Team Asterix — Self-Driving Buggy Software

A plain-English guide to everything inside the Team-Asterix-Lane-Keep folder

Folder: C:\Users\HP\Team-Asterix-Lane-Keep      Total size: about 275 MB      Prepared: 24 September 2026

# 1. What is this folder, in one paragraph?

This folder holds the software for a student-built off-road racing buggy (an SAE BAJA vehicle) that can drive itself. Two things are being taught to the vehicle. First, staying inside its lane by looking through a camera and turning the steering — this is called LKA (Lane Keep Assist). Second, stopping by itself before it hits something in front of it — this is called AEB (Automatic Emergency Braking). Some of the code runs on the real buggy with real sensors and a real Arduino board; some of it runs inside a driving simulator on a computer for a competition submission.

Nothing here is a toy example. It is real working code with real camera recordings, real log files, real motor commands and real safety checks.

## The three projects inside

| # | Folder | What it is | Runs on |
| --- | --- | --- | --- |
| 1 | the main folder (root) | Lane Keep Assist. A camera watches the road, a neural network finds the lane lines, and the software works out how much to turn the steering wheel. It sends that angle to an Arduino board. | Real vehicle (NVIDIA Jetson computer + Arduino) |
| 2 | Team-Asterix-AEB | Automatic Emergency Braking. A radar and a depth camera watch ahead, their readings are merged, and the software decides CLEAR, CAUTION or BRAKE, then commands the brakes. | Real vehicle (Jetson + CAN radar + ZED camera + Arduino) |
| 3 | Team-Asterix---Software-Submission | The official competition entry for aBAJA SAEINDIA 2026. Same ideas, but rewritten for the IPG CarMaker simulator using ROS 2 and Simulink. Also contains the reports, results and evidence. | A PC running CarMaker + MATLAB/Simulink + ROS 2 |

Note:  Projects 1 and 2 are the real hardware. Project 3 is the simulation version that gets submitted for marks. They share the same engineering ideas but are separate pieces of code.

# 2. The folder map

Here is the whole folder laid out. Only the important items are shown.

Team-Asterix-Lane-Keep/
│
├── README.md, LKA_README.md, BENCH.md, BRIDGE.md   <- the four guide documents
├── requirements.txt                                <- list of Python libraries needed
│
├── run_video_annotated.py        <- MAIN script: video/camera in, steering out + overlay video
├── run_video_annotated - v2.py   <- a newer, bigger version of the same script
├── run_video_to_arduino.py       <- simpler script: video in, steering straight to Arduino
├── bench_steering_to_arduino.py  <- workbench test tool for the steering wires
├── steer_bench.py                <- safe round-trip test of the steering link
├── single_lane_stereo_recovery.py<- rescue maths for when only one lane line is visible
├── export_yolop_onnx.py          <- converts the AI model to a faster format
├── arduinocode.ino               <- the program that runs ON the Arduino board (C++)
│
├── src/                          <- the reusable engine of the lane-keeping system
│   ├── perception/               <- "what do I see?"  (camera -> lane position)
│   │   └── lane_core/            <- 13 small files, one job each
│   ├── signal_conditioning/      <- throws away obviously wrong readings
│   ├── ego_state/                <- "how fast am I going, how am I turning?"
│   ├── reference/                <- the tidy hand-over package between seeing and steering
│   ├── safety/                   <- the referee: decides if auto-steering is allowed
│   └── actuator/                 <- "send the steering command to the hardware"
│
├── YOLOP/                        <- the borrowed AI model that finds lane lines (124 MB)
│   ├── lib/                      <- the model's code (trimmed to only what is needed)
│   └── weights/                  <- the trained model files (.pth, .onnx, .engine)
│
├── tests/                        <- automated check of the lane-holding safety gate
│
├── Team-Asterix-AEB/             <- PROJECT 2: radar + camera emergency braking
│   ├── main.py, pipeline.py      <- entry point and the glue
│   ├── radar_processing.py       <- Module 1: radar
│   ├── camera_processing.py      <- Module 2: ZED depth camera
│   ├── sensor_fusion.py          <- Module 3: Kalman filter that merges the two
│   ├── aeb_decision.py           <- Module 4: CLEAR / CAUTION / BRAKE and the brake command
│   ├── data_logger.py, latency.py, telemetry_logger.py, simulate.py
│   ├── remote_server.py + templates/  <- phone web remote control
│   ├── logs/                     <- recorded runs (.jsonl and .csv)
│   ├── tests/                    <- one-sensor-at-a-time hardware checks
│   └── or/orcode/                <- the older version of the same project, kept for reference
│
└── Team-Asterix---Software-Submission/   <- PROJECT 3: competition submission (145 MB)
    ├── src/AEB/aeb_controller/   <- ROS 2 package: radar tracker, AEB node, arbitration
    ├── src/LKA/lane_detection/   <- ROS 2 package: classic OpenCV lane finder
    ├── src/LKA/lane_node/        <- ROS 2 package: publishes lane state
    ├── src/LKA/lateral_controller/ <- ROS 2 package: turns lane state into steering
    ├── src/simulink/             <- the Simulink vehicle model (.mdl, 762 KB)
    ├── config/                   <- how the simulated sensors are set up
    ├── docs/                     <- 3 PDF reports (architecture, sensors, hardware)
    ├── results/                  <- 477 debug images, run logs, result PDFs
    └── Submission_Guidelines.md  <- the competition rulebook

# 3. Project 1 — Lane Keep Assist (the main folder)

This is the biggest and most polished part. Its job is simple to say and hard to do: keep the buggy in the middle of its lane by turning the steering, and never touch the throttle or the brakes. A human safety driver still controls the speed and can stop at any moment.

## 3.1 How it works, step by step

Every single camera frame goes through the same chain of steps:

| Step | What happens | Why it matters |
| --- | --- | --- |
| 1. Get the picture | A ZED stereo camera gives a wide 3840x1080 image; the software crops out just the left eye. | The rest of the pipeline only needs one normal camera view. |
| 2. Undistort | A camera-lens correction step (camera_calibration.py). | Currently a pass-through — no real checkerboard calibration has been done yet, and the code says so honestly. |
| 3. Brighten if dark | If the frame is genuinely dark, gamma + CLAHE brightening is applied (adaptive_enhancement.py). | Helps in dim light. Bright frames are left completely untouched. |
| 4. AI lane detection | The YOLOP neural network marks, for every pixel, how likely it is to be a lane line (model_runtime.py). | This replaces the old-fashioned edge-detection approach and is far more robust. |
| 5. Ignore the sky | The top 35% of the AI's output is wiped to zero (roi_mask.py). | Tree lines, buildings and glare can never be mistaken for a lane. |
| 6. Bird's-eye view | The image and the AI map are warped so the road looks like it is seen from directly above (ipm.py). | In this view, lane lines are parallel and distances behave sensibly. |
| 7. Find the two edges | Left and right lane boundary pixels are pulled out of the warped map (bird_eye_extraction.py). | Uses quality gates: a candidate must be long enough and reach the vehicle. |
| 8. Colour sanity check | Candidates are checked against the original colour image — is it actually bright white paint? (color_filter.py) | Fixes a real, measured bug where a dirt shoulder or a kerb was being picked as a lane line. |
| 9. Fit a curve | A second-order (quadratic) curve is fitted through the points (curves.py). | Turns thousands of pixels into three numbers per lane edge. |
| 10. Rebuild a missing edge | If only one side is visible, the other is reconstructed using a learned lane width (calibration.py). | The buggy keeps working when one line disappears in a turn. |
| 11. Work out the error | Two numbers come out: cross-track error (how far off-centre, in metres) and heading error (how crooked, in degrees). | These two numbers are all the steering controller needs. |
| 12. Reject nonsense | A gate checks confidence and whether the numbers jumped further than physically possible (conditioner.py). | A single garbage frame cannot poison the filter. |
| 13. Smooth over time | A Kalman filter tracks the four values [offset, offset rate, heading, heading rate] (kalman_tracker.py). | Removes jitter and can coast through a few missed frames. |
| 14. Decide the steering | The Stanley steering law converts the two errors into a steering angle, clamped to +/-30 degrees (steering_v2.py). | A well-known, proven self-driving steering formula. |
| 15. Limit how fast it moves | A slew-rate limiter stops the command jumping too quickly. | Protects the physical steering mechanism. |
| 16. Safety referee | SafetyMonitor decides ENGAGED or not (safety_monitor.py). | If anything is wrong, zero steering is sent instead. |
| 17. Send it | The angle is averaged over one second and sent to the Arduino as a text line (arduino_steering_link.py). | One second of averaging is the single biggest anti-wobble measure. |

## 3.2 The main scripts you actually run

| File | Lines | What it does |
| --- | --- | --- |
| run_video_annotated.py | 878 | The main tool, called 'the bridge'. Takes any video file OR a live camera, runs the whole pipeline, and produces (a) a video with the lane drawn on it, a bird's-eye inset and a dashboard readout, and (b) a CSV file with one steering command per second. Works with no hardware attached at all. Add --steer-port to also drive the real Arduino. |
| run_video_annotated - v2.py | 1100 | A newer, larger version of the same script with more features. Note the space in the filename — it must be quoted on the command line. |
| run_video_to_arduino.py | 216 | The stripped-down version: video file in, steering commands out to a real Arduino over USB. No video output, logs every frame. Requires a serial port to be connected. |
| bench_steering_to_arduino.py | 332 | A workbench test rig. Sends test steering commands and prints everything sent and received, so the terminal becomes your serial monitor. Has a sweep mode, a manual-typing mode, a CSV replay mode and a live-video mode. Described as temporary and deletable. |
| steer_bench.py | 121 | A safer round-trip proof. It forces the Arduino into dry-run (no physical movement), confirms that from telemetry, then checks each steering target comes back correctly. It refuses to move real wheels unless you explicitly pass --allow-live. |
| single_lane_stereo_recovery.py | 375 | Special maths for sharp turns, when one lane line leaves the frame. Instead of guessing in image space, it uses the ZED camera's real 3D depth to work out the true position of the one line it can still see. |
| export_yolop_onnx.py | 64 | A one-time conversion tool. Exports only the lane-finding part of the AI model to ONNX format so it can be turned into a much faster TensorRT engine on the Jetson. |
| arduinocode.ino | 1084 | Not Python — this is the C++ program that is uploaded to the Arduino board itself. It receives the text commands, drives a stepper motor for steering and a DAC chip for throttle, handles brake, does a steering-pot calibration routine, saves settings to EEPROM, and sends telemetry back about 20 times a second. |

## 3.3 Inside src/ — the reusable engine

The src folder is split by responsibility. Each folder answers one question.

### src/perception — 'what do I see?'

| File | Lines | Job in one sentence |
| --- | --- | --- |
| lane_detection_node_birdeye.py | 663 | The conductor. Runs steps 1 to 15 above in order and hands back the result for each frame. |
| lane_core/model_runtime.py | 124 | Loads YOLOP and runs one frame through it. |
| lane_core/camera_calibration.py | 52 | Lens undistortion hook (currently pass-through). |
| lane_core/adaptive_enhancement.py | 105 | Brightens only genuinely dark frames. |
| lane_core/roi_mask.py | 25 | Blanks the sky region of the AI output. |
| lane_core/ipm.py | 123 | Warps the view to bird's-eye. |
| lane_core/bird_eye_extraction.py | 160 | Pulls left/right lane pixels out of the warped map. |
| lane_core/color_filter.py | 53 | Confirms a candidate really is white paint. |
| lane_core/lane_continuity.py | 24 | Uses last frame's curve to predict where this frame's should be. |
| lane_core/curves.py | 59 | Fits the quadratic curve. |
| lane_core/calibration.py | 74 | Learns lane width so a missing side can be rebuilt. |
| lane_core/kalman_tracker.py | 169 | The Kalman filter that smooths offset and heading. |
| lane_core/steering_v2.py | 113 | The Stanley steering formula plus the slew-rate limiter. |

### The other five src folders

| Folder / file | Lines | Job in one sentence |
| --- | --- | --- |
| signal_conditioning/conditioner.py | 72 | A gate in front of the Kalman filter. Rejects a frame if confidence is too low, or if the numbers moved further than the vehicle physically could have. |
| ego_state/ego_state_estimator.py | 66 | Short-range dead reckoning: 'how far have I moved since the last camera update?' so the system can bridge a dropped frame. |
| ego_state/imu_source.py | 310 | Plug-in points for real motion sensors: a WitMotion HWT905 IMU for turn rate, and the Arduino for speed. Right now a NullImuSource is active, meaning no sensor is wired and the system degrades gracefully. |
| reference/reference_generator.py | 50 | The tidy hand-over package between 'seeing' and 'steering'. Keeps the downstream code from reaching into the perception node's internals. |
| safety/safety_monitor.py | 107 | The referee. Four duties: check the command is physically plausible using a bicycle model, watch for stale sensors, allow a driver override, and run a command watchdog. |
| actuator/arduino_steering_link.py | 237 | The correct link for the real firmware. Sends STA<angle> only — never throttle or brake — and reads $TEL telemetry back in a background thread to get the vehicle's real speed. |
| actuator/sda_serial_backend.py | 130 | An older link that speaks SDA <value>. Documented as wrong for the current Arduino sketch, which would misread it as a throttle command. |
| actuator/dbw_interface.py | 90 | The general 'drive-by-wire' actuator interface with a second rate limiter and a watchdog. |
| actuator/dbw_backend.py | 123 | Pluggable hardware back ends. NullDbwBackend (logs only, touches nothing) is what actually runs on a dev machine. |

## 3.4 Engagement — when is auto-steering actually allowed?

The Arduino only receives a real, non-zero steering command when the safety monitor says ENGAGED. All of these must be true at the same time, for 5 frames in a row:

Speed is between 0.5 and 15.0 metres per second.

Lane-tracking confidence is above 0.3.

The Kalman tracker is started and not stale.

Commands are still flowing (the actuator watchdog has not tripped).

No other fault is active.

If any of those fails, an explicit neutral command (zero degrees) is sent instead of going silent. Going silent would look like a dead link and would trip the watchdog, which would then make it impossible to ever engage.

## 3.5 The four guide documents

| Document | What it covers |
| --- | --- |
| README.md | Setup on the Jetson: installing NVIDIA's special PyTorch build, serial-port permissions, the TensorRT speed-up steps, every command line flag, and the SDA wire protocol. |
| LKA_README.md | The operator's guide for an actual drive: the STA protocol, one-time steering-pot calibration, a bench dry-run procedure, a pre-drive checklist, what to do if the steering wobbles, and a troubleshooting table. |
| BENCH.md | How to test the Jetson-to-Arduino steering link on a workbench and confirm the direction sign is correct before anything moves. |
| BRIDGE.md | Explains the run_video_annotated.py tool, every column in its CSV output, and how the two separate lane-keeping code bases relate to each other. |

## 3.6 Honesty in the code — worth noticing

The comments in this project are unusually frank about what is not yet proven. Real examples found in the files:

Camera lens calibration has never been done — the file says so and passes frames straight through.

There is no real speed sensor, so speed is an operator-supplied number unless the Arduino's telemetry is live.

The bird's-eye view is uncalibrated, so curvature is proportional to reality, not measured in real units.

The Kalman filter's noise settings are described as reasonable starting points, not bench-tuned values.

A known mismatch is documented: the firmware treats 24 degrees as full lock, but the software allows up to 30, so commands between 24 and 30 saturate.

Note:  Documenting limitations like this is a sign of good engineering practice, and it is exactly what the competition rules ask teams to do.

# 4. Project 2 — Team-Asterix-AEB (emergency braking on the real buggy)

This project answers a different question: is there something in front of me, how far away, and should I brake? It uses two sensors that see the world in completely different ways, then merges them.

## 4.1 The two sensors

| Sensor | What it is | Strengths | Weaknesses |
| --- | --- | --- | --- |
| Continental ARS408 radar | An automotive radar on a CAN bus at 500 kbit/s. Its own internal tracker reports objects with stable IDs, velocity, a confidence score and a class (pedestrian, car, truck). | Works in dust, rain and darkness. Measures speed directly. | It is 2D — it cannot tell height, so an overhead branch or a ground bump can look like an obstacle. |
| ZED stereo camera | A depth camera. Two lenses give a real distance for every pixel. | Gives true 3D — height, sideways offset and forward distance. | Struggles in dust and bad light. |

Each sensor covers the other's blind spot. That is the whole reason for fusing them: the camera's height information is used to throw out the radar returns that are actually tree branches.

## 4.2 The four modules

The project was deliberately refactored from one big file into four independent modules, so each one can be tested completely on its own with no hardware attached.

| Module | File | Lines | What it owns |
| --- | --- | --- | --- |
| 1 — Radar | radar_processing.py | 112 | Reading the radar over CAN, reporting radar health, and filtering down to the single closest object inside the driving corridor. |
| 2 — Camera | camera_processing.py | 397 | Grabbing depth from the ZED, correcting for where it is mounted relative to the radar, converting pixels to real 3D positions, and judging whether something is overhead and should be ignored. |
| 3 — Fusion | sensor_fusion.py | 162 | Pure maths: a Kalman filter that turns the radar distance and the camera distance into one trusted distance. No hardware, no CAN bus. |
| 4 — Decision | aeb_decision.py | 368 | Takes the fused distance and decides CLEAR, CAUTION or BRAKE, then sends the matching speed or brake command to the Arduino. Also cross-checks against wheel speed. |

The braking rule itself is simple and readable:

The object must be in front (0.3 m to the caution range).

It must be inside the vehicle's path corridor (sideways offset within half the vehicle width plus a margin).

Its confidence must be above the threshold.

10 to 15 metres away gives CAUTION; 0.3 to 10 metres gives BRAKE.

Debouncing ignores single-frame ghosts; a short latch keeps BRAKE on through a momentary dropout.

## 4.3 The supporting files

| File | Lines | What it does |
| --- | --- | --- |
| main.py | 601 | The program you actually run on the vehicle. Handles the command-line options and the run modes. |
| pipeline.py | 119 | Pure glue — calls the four modules in order each cycle. Contains almost no logic of its own on purpose. |
| data_logger.py | 120 | Writes one JSON line per cycle recording everything: raw radar scan, camera distance, filter state, decision, actuator command and timings. JSONL format so a crash loses at most one line. |
| latency.py | 139 | Times each stage with a high-resolution clock and keeps rolling average, maximum and worst-case figures. |
| simulate.py | 196 | Replays a recorded log through the real fusion and decision code. No radar, no camera, no Arduino needed — perfect for debugging one incident afterwards. |
| telemetry_logger.py | 33 | Writes the Arduino's own telemetry (RPM, steering, throttle) to a CSV. |
| remote_server.py + templates/index.html | 420 + web page | A small Flask web server so a phone on the same Wi-Fi can send speed commands to the Arduino. It has a dead-man's switch: if the phone stops sending a heartbeat, the vehicle is stopped. |
| tests/ | 5 files | One-sensor-at-a-time hardware checks: check_camera.py, check_radar.sh, comm_check.py, serial_monitor_test.py, and test_all.sh which runs all of them. |
| logs/ | 4 files | Recorded runs from 14 July 2026 — two JSONL cycle logs and two telemetry CSVs. |
| or/orcode/ | 20 files | The older version of this project, kept for reference. Contains the original radar.py (507 lines, the CAN decoder and live bird's-eye visualiser) and aeb.py (555 lines, the decision evaluator), which the new code still imports unchanged. |

## 4.4 A clear safety stance

The README is blunt about it. The AEB is described as a prototype driver-assist alert. Commands are only actually sent when you pass --arm. There is a --dry-run mode, the radar configuration is sent as volatile so a power cycle restores factory defaults, and the documentation repeatedly says to bench test with the wheels off the ground and to keep a human on a physical kill switch.

# 5. Project 3 — the competition submission

This folder is the official entry for the aBAJA SAEINDIA 2026 Software Virtual World Simulation event. It is its own separate Git repository nested inside the main one. The team number is 264016, which is why every file name starts with it.

## 5.1 What the competition asks for

| Event | What the team controls | Status in this folder |
| --- | --- | --- |
| AEB | Throttle and brake only | Code present; results folder still empty. |
| LKA | Steering only | Fully done — 477 debug images across 15 runs, a run log CSV and an observations PDF. |
| Endurance | Throttle, brake and steering together | Code present; results folder still empty. |
| Self-created scenario | Team's own test | Report PDF present. |

Key rules the design has to respect, straight from the guidelines file:

No using the simulator's ground-truth object positions as inputs — perception must be genuine.

No hard-coding scenario positions or replaying recorded data.

Everything must be reproducible on a real vehicle.

Perception must run at 10 Hz or better; control at 20 Hz or better.

## 5.2 The ROS 2 packages

Unlike Projects 1 and 2, which are plain Python scripts, this one is built as proper ROS 2 packages — the standard robotics framework where separate programs (called nodes) talk to each other by publishing messages on named topics.

| Package / file | Lines | What it does |
| --- | --- | --- |
| LKA/lane_detection/...lane_detection.py | 868 | The lane finder, written the classic computer-vision way instead of with a neural network: strip the black border, crop to the road, warp to bird's-eye, threshold in HLS colour space plus Sobel edges, sliding-window pixel search, RANSAC polynomial fit, then output cross-track error, heading error and curvature, smoothed over time. |
| LKA/lane_node/...lane_node.py | 409 | The ROS 2 node. Subscribes to /cameraImages, publishes /perception/lane_state, plus an optional debug image. |
| LKA/lateral_controller/...lateral_controller.py | 112 | Reads lane state, computes the steering, publishes /vehicleControl. |
| AEB/aeb_controller/...radar_tracker.py | 305 | A multi-object Kalman tracker for the simulated radar. Subscribes to /RadarTracks, publishes /perception/tracked_objects. |
| AEB/aeb_controller/...aeb_node.py | 163 | The braking node — works out time-to-collision and publishes a brake command. |
| AEB/aeb_controller/...aeb_manager.py | 324 | The arbitrator sitting above both. LKA asks for steering, AEB asks for braking, and this node decides who wins — braking always beats lane keeping. |

Each package has the standard ROS 2 paperwork: package.xml (declaring dependencies on rclpy, std_msgs, geometry_msgs, radar_msgs and a custom adas_msgs), setup.py, setup.cfg, and three style tests (copyright, flake8 and PEP 257 docstring checks).

## 5.3 The simulator side

config/CarMaker_Sensor_Config.md documents exactly how the virtual vehicle and its sensors are set up:

| Item | Setting |
| --- | --- |
| Simulator | IPG CarMaker, driven from Simulink R2024b via the CarMaker4SL blockset |
| Camera | 640 x 480 RGB, arriving over TCP on localhost port 2210, then republished on the /cameraImages ROS topic |
| Radar | CarMaker radar sensor, published as radar_msgs/RadarTracks on /RadarTracks |
| Vehicle state read | speed, distance travelled, lateral position on road, and all four wheel speeds |
| Wheelbase | 1.45 m (Baja SAE platform) |
| In-model controller | Stanley — the same steering law as the real vehicle project |
| Control topic | /vehicleControl, where x = steering, y = throttle, z = brake |

src/simulink/264016_Asterix_Simulink.mdl (762 KB) is the Simulink model itself — the block diagram that connects CarMaker to the ROS 2 stack.

## 5.4 The evidence folder

| What | Where | Contents |
| --- | --- | --- |
| Reports | docs/ | Three PDFs: Hardware Report, Sensor Config and System Architecture. |
| LKA evidence | results/LKA/ | 477 annotated debug frames spread over 15 separate runs from 12 May 2026, plus LKA_Observations.pdf. |
| LKA data | results/LKA/Data/Logs/lka_run.csv | A per-tick log with columns: time, lateral error in metres, steering command in degrees, running RMSE, an iterative-learning-control term, and a lane-detected flag. |
| Self-created scenario | results/Self_Created/ | The team's own scenario report PDF. |
| Still to do | results/AEB/, results/Endurance/ | Both are empty except for placeholder files. |

# 6. Every technology used, and why

## 6.1 Programming languages

| Language | Where it is used | Why |
| --- | --- | --- |
| Python 3 | Almost everything — all three projects | Fast to write, huge library support for vision and maths, and it is explicitly allowed by the competition rules. |
| C++ (Arduino sketch) | arduinocode.ino, 1084 lines | The Arduino board runs bare-metal C++. This is the layer that physically moves the stepper motor and the brake. |
| MATLAB / Simulink | src/simulink/...mdl | The simulator's vehicle model and the in-model controller are built as Simulink block diagrams. |
| Bash | check_radar.sh, test_all.sh, radar_canup.sh | Small shell helpers for bringing the CAN bus up and running hardware checks. |
| HTML / CSS / JavaScript | Team-Asterix-AEB/templates/index.html | The phone remote-control page. |

## 6.2 Libraries and frameworks

| Technology | What it is | What it does here |
| --- | --- | --- |
| PyTorch | The deep-learning framework | Loads and runs the YOLOP neural network. On the Jetson it must be NVIDIA's special JetPack build, not the normal one from PyPI — the README explains this at length. |
| YOLOP | A published multi-task driving network | Marks lane-line pixels. Only its lane head is used; the object-detection and drivable-area heads are ignored. The copy here is trimmed to just what inference needs — 1238 lines instead of the full project. |
| OpenCV (cv2) | The standard computer-vision library | Reading video, warping to bird's-eye, CLAHE brightening, drawing the overlays, and the whole classic lane pipeline in the submission project. |
| NumPy | Array and matrix maths | Every numerical step — curve fitting, the Kalman filter, the geometry. |
| TensorRT | NVIDIA's model optimiser | Optional but recommended. Converts the model into a hardware-specific engine. The README measures the gain: about 113 ms per frame with plain PyTorch, down to roughly 15-35 ms with TensorRT — a 3 to 6 times speed-up. |
| ONNX | A neutral model file format | The stepping stone between PyTorch and TensorRT. |
| pyserial | Serial-port library | Every Jetson-to-Arduino conversation goes through it. |
| python-can / cantools | CAN bus libraries | Talking to the ARS408 radar in Project 2. |
| ZED SDK | Stereolabs' stereo-camera library | Gives real metric depth for every pixel — used for the one-lane recovery maths and for the AEB camera module. |
| ROS 2 (rclpy) | The robotics middleware | The whole competition submission is built as ROS 2 nodes publishing and subscribing to topics. |
| Flask | A small Python web framework | Serves the phone remote-control page. |
| Matplotlib | Plotting library | Draws the live radar bird's-eye map and the heatmap. |
| PyYAML / yacs | Configuration file readers | YOLOP's own configuration loader needs them. |
| Pillow | Image library | Image handling support for PyTorch transforms. |
| IPG CarMaker | A physics-accurate vehicle simulator | The virtual world the competition entry drives in. |

## 6.3 Hardware

| Hardware | Role |
| --- | --- |
| NVIDIA Jetson (Orin class) | The on-board computer. Runs the AI model and all the Python. |
| ZED stereo camera | The eyes. 3840 x 1080 side-by-side; the software uses the left eye for lane finding and the depth map for 3D work. |
| Continental ARS408 radar | Forward obstacle detection over CAN at 500 kbit/s. |
| Arduino board | The low-level controller. Drives the steering stepper, a MCP4725 DAC for throttle and the brake, and reports telemetry back. |
| WitMotion HWT905 IMU | Turn-rate sensor. Support code exists but it is not currently wired up. |
| Stepper motor + steering pot | Physically turns the wheels, with a potentiometer for position feedback. |

## 6.4 Algorithms and engineering methods

| Method | In plain words | Used where |
| --- | --- | --- |
| Semantic segmentation | A neural network labels every single pixel as lane or not-lane. | Project 1 perception |
| Inverse perspective mapping | Warping the camera view so the road looks like it is seen from a helicopter directly above. | Project 1 and Project 3 |
| Sliding-window search | Walking up the image in blocks, following the lane pixels. | Project 3 lane detection |
| Polynomial curve fitting | Drawing the smooth curve that best matches a cloud of points. | Both lane pipelines |
| RANSAC | A fitting method that ignores outliers, so a few stray pixels cannot bend the whole curve. | Project 3 lane detection |
| Kalman filter | A principled way of blending a noisy new measurement with what you already believed, and of coasting sensibly when no measurement arrives. | Lane tracking, radar object tracking, and radar/camera fusion |
| Stanley steering law | A proven self-driving formula that turns 'how far off-centre' and 'how crooked' into a steering angle, automatically steering more gently at speed. | All three projects |
| Slew-rate limiting | Never let the command change faster than the hardware can follow. | Project 1, twice over |
| Bicycle model | A simple vehicle-physics model used to check that a commanded steering angle is physically sensible at the current speed. | Project 1 safety monitor |
| Sensor fusion | Combining two sensors so each covers the other's weakness. | Project 2 |
| Time-to-collision | Distance divided by closing speed — how many seconds until impact. | Project 3 AEB node |
| Watchdog timers | If an expected signal stops arriving, assume something has broken and fail safe. | Everywhere |
| Debounce and latch | Ignore a single-frame blip, but hold a real alarm through a brief dropout. | Project 2 AEB |
| Dead reckoning | Estimating how far you have moved from speed and turn rate alone, with no camera. | Project 1 ego state |

## 6.5 Wire protocols — how the parts talk

| Protocol | Direction | Format and meaning |
| --- | --- | --- |
| STA | Jetson to Arduino | STA<angle> — an absolute steering angle. Positive is right, negative is left. This is the correct, current protocol. |
| $TEL | Arduino to Jetson | A comma-separated telemetry line about 20 times a second: time, state, speed in km/h, wheel RPM, throttle percent, actual and target steering angle, brake values, PID error, fault flags and the dry-run flag. |
| SDA | Jetson to Arduino | SDA <angle to 2 decimals> — an older protocol. The documentation warns clearly that the current firmware would misread it as a throttle command. |
| STR / STL / STC | Jetson to Arduino | Steer right, steer left, steer centre — used by the bench test tool for a different sketch. |
| CAN frames | Radar to Jetson | Raw automotive CAN. Frame 0x60A marks a cycle boundary, 0x60B carries position and velocity, 0x60C carries confidence, 0x60D carries class and size. 0x200 is sent by the software to choose the radar's output mode. |
| ROS 2 topics | Between simulator and stack | /cameraImages and /RadarTracks come in; /perception/lane_state and /vehicleControl go out. |

# 7. How to actually run things

These are taken directly from the project's own documentation.

### Lane keeping — no hardware needed

python run_video_annotated.py --source myvideo.mp4

Produces an annotated video and a per-second steering CSV next to the input file. Nothing is connected, nothing moves.

### Lane keeping — on the vehicle

python run_video_annotated.py \
    --source 0 \
    --stereo-eye left \
    --adaptive-ipm \
    --steer-port /dev/ttyUSB0

--source 0 means the live camera. --stereo-eye left crops the ZED's side-by-side image. --adaptive-ipm auto-tunes the bird's-eye framing. --steer-port is the steering-only link to the Arduino.

### Bench test the steering wiring first

python3 bench_steering_to_arduino.py --sweep

Always do this before anything else. It proves the wiring works and that a 'right' command genuinely turns the wheels right.

### Emergency braking on the vehicle

python3 main.py --print --arm

--arm is what allows real brake commands. Leave it off to watch the system think without anything moving.

### Replay a recorded AEB run with no hardware

python3 simulate.py --log logs/aeb_2026-07-14_12-26-48.jsonl --print

### Run the lane-gate unit tests

python tests/test_lane_hold_gate.py

152 lines of plain assertions — no test framework needed, and it does not load the AI model.

# 8. Quick reference — the numbers

| Measure | Value |
| --- | --- |
| Total folder size | about 275 MB |
| Project 1 (lane keeping) code | about 208 KB of Python across 23 modules |
| The AI model files | 124 MB (92 MB PyTorch, 20 MB ONNX, 12 MB TensorRT engine) |
| Project 2 (AEB) | 5.9 MB, 4 core modules plus tools and an older version |
| Project 3 (submission) | 145 MB, mostly the 477 result images |
| Arduino firmware | 1084 lines of C++ |
| Largest Python file | run_video_annotated - v2.py at 1100 lines |
| Steering limit in software | plus or minus 30 degrees |
| Steering limit in firmware | about 24 degrees is full lock (a known mismatch) |
| Engage speed window | 0.5 to 15.0 m/s |
| Minimum confidence to engage | 0.3 |
| Frames of good data needed to engage | 5 in a row |
| Steering averaging window | 1 second, re-sent every 0.5 seconds |
| AEB caution zone | 10 to 15 metres |
| AEB brake zone | 0.3 to 10 metres |
| Speed with plain PyTorch | about 113 ms per frame, roughly 9 frames per second |
| Speed with TensorRT | about 15 to 35 ms per frame, roughly 25 to 40 frames per second |
| Vehicle wheelbase | 1.45 metres |
| Radar CAN bit rate | 500 kbit/s |
| Arduino telemetry rate | about 20 times per second |

# 9. Glossary — the jargon, decoded

| Term | Plain meaning |
| --- | --- |
| LKA | Lane Keep Assist — steering itself to stay in the lane. |
| AEB | Automatic Emergency Braking — stopping itself before hitting something. |
| Cross-track error (CTE) | How far sideways the vehicle is from the middle of the lane, in metres. |
| Heading error | How crooked the vehicle is compared to the lane direction, in degrees. |
| Bird's-eye / IPM | The warped top-down view of the road. |
| Stanley controller | The formula that turns those two errors into a steering angle. |
| Kalman filter | Maths that smooths noisy readings and fills in short gaps. |
| Segmentation | A neural network labelling every pixel. |
| Slew rate | How fast a command is allowed to change. |
| Watchdog | A timer that raises an alarm if an expected signal stops. |
| Engaged | The state where auto-steering is actually allowed to act. |
| Dry run | A mode where all the maths runs but no hardware moves. |
| Serial port | The USB cable conversation between the Jetson and the Arduino. |
| CAN bus | The wiring standard that cars use for sensors to talk to computers. |
| ROS 2 | A robotics framework where separate programs talk over named topics. |
| Jetson | NVIDIA's small computer with a built-in GPU, made for robots. |
| TensorRT | NVIDIA's tool for making a neural network run much faster on their hardware. |
| Telemetry | The stream of status numbers the Arduino sends back. |
| JSONL | A log format with one complete JSON record per line, so a crash only loses the last line. |
| Corridor | The strip of road directly ahead that the vehicle will actually drive through. |

# 10. Summary

Three related bodies of work live in this one folder.

A lane-keeping system for a real buggy, built around a YOLOP neural network, a bird's-eye transform, a Kalman filter and a Stanley steering controller, with a genuine safety layer that decides when auto-steering is allowed and an Arduino that physically turns the wheels.

An emergency-braking system that merges an automotive radar with a stereo depth camera so each covers the other's weakness, split into four independently testable modules, with full logging and an offline replay tool.

A competition submission that reimplements both ideas inside the CarMaker simulator using ROS 2 and Simulink, together with the reports, run logs and 477 debug frames that serve as evidence.

Two things stand out across all of it. The documentation is unusually thorough — four full guides for the lane system alone, with pre-drive checklists and troubleshooting tables. And the code is honest: every placeholder, every untested path, every hand-picked value that has not been properly tuned is labelled as such in the comments, rather than being quietly passed off as finished.